RGB LED Control System with LCD Menu

Aditya Maindargi  |  Manufacturing Science & Engineering

COEP Technological University

# Overview

This project is an interactive RGB LED control system built using an Arduino UNO, I2C LCD display, potentiometer, push button, and RGB LED.

The system allows the user to navigate between different color options using a potentiometer. The currently selected color is displayed on the LCD screen with a menu-style selector (>). Pressing the button turns ON the selected LED color, while releasing the button turns the LED OFF.

The entire project was:

- Designed logically from scratch

- Simulated in Tinkercad

- Physically wired and tested

- Documented and uploaded to GitHub

# Features

- Interactive LCD menu system

- RGB LED color selection

- Potentiometer-based navigation

- Push-button controlled output

- I2C LCD communication

- Modular function-based programming

- Real-time hardware interaction

# Components Used

- Arduino UNO

- 16x2 I2C LCD Display

- RGB LED

- Potentiometer

- Push Button

- Resistors

- Breadboard

- Jumper Wires

# Circuit Connections

## RGB LED Connections

| RGB LED Pin | Arduino Pin |
| --- | --- |
| Red | D11 |
| Green | D10 |
| Blue | D9 |

## Potentiometer Connections

| Potentiometer Pin | Connection |
| --- | --- |
| Left Pin | 5V |
| Middle Pin | A0 |
| Right Pin | GND |

## Push Button Connections

| Button Pin | Connection |
| --- | --- |
| One Side | D8 |
| Other Side | GND |
| Pull-down Resistor | Connected to GND |

## I2C LCD Connections

| LCD Pin | Arduino UNO |
| --- | --- |
| GND | GND |
| VCC | 5V |
| SDA | A4 |
| SCL | A5 |

# Simulation Screenshots

Image 1 - Red selected, cursor on row 1

Image 2 - Red selected, cursor on row 2

Image 3 - Green selected, Blue below

Image 4 - Green selected, Blue highlighted

# Working Principle

- The potentiometer is used to navigate through color options.

- The LCD displays the current selection.

- A selector (>) indicates the active choice.

- When the push button is pressed: the selected RGB color turns ON.

- When the button is released: the RGB LED turns OFF.

## LCD Menu Logic

The menu dynamically changes based on potentiometer values:

- Red selected

- Green selected

- Blue selected

The LCD updates in real-time according to analog input.

# Source Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

LiquidCrystal_I2C lcd(0x27, 16, 2);

int Pot = A0;

int PotVal;

int Red = 11;

int Green = 10;

int Blue = 9;

int button = 8;

int button_input;

String Color[] = {"Red", "Green", "Blue"};

void red() {

digitalWrite(Red, HIGH);

digitalWrite(Green, LOW);

digitalWrite(Blue, LOW);

}

void green() {

digitalWrite(Red, LOW);

digitalWrite(Green, HIGH);

digitalWrite(Blue, LOW);

}

void blue() {

digitalWrite(Red, LOW);

digitalWrite(Green, LOW);

digitalWrite(Blue, HIGH);

}

void Light() {

if (PotVal <= 330) { red(); }

else if (PotVal <= 679) { green(); }

else { blue(); }

}

void Light_off(){

digitalWrite(Red, LOW);

digitalWrite(Green, LOW);

digitalWrite(Blue, LOW);

}

void setup() {

pinMode(Red, OUTPUT);

pinMode(Green, OUTPUT);

pinMode(Blue, OUTPUT);

pinMode(Pot, INPUT);

pinMode(button, INPUT);

Serial.begin(9600);

lcd.init();

lcd.backlight();

}

void loop() {

PotVal = analogRead(Pot);

button_input = digitalRead(button);

Serial.println(PotVal);

lcd.clear();

if (PotVal <= 330) {

lcd.setCursor(0, 0); lcd.print(">");

lcd.setCursor(3, 0); lcd.print(Color[0]);

lcd.setCursor(3, 1); lcd.print(Color[1]);

} else if (PotVal <= 580) {

lcd.setCursor(0, 1); lcd.print(">");

lcd.setCursor(3, 0); lcd.print(Color[0]);

lcd.setCursor(3, 1); lcd.print(Color[1]);

} else if (PotVal <= 680) {

lcd.setCursor(0, 0); lcd.print(">");

lcd.setCursor(3, 0); lcd.print(Color[1]);

lcd.setCursor(3, 1); lcd.print(Color[2]);

} else {

lcd.setCursor(3, 0); lcd.print(Color[1]);

lcd.setCursor(0, 1); lcd.print(">");

lcd.setCursor(3, 1); lcd.print(Color[2]);

}

if (button_input == 0) { Light(); }

else if (button_input == 1) { Light_off(); }

delay(150);

}

# Technologies Used

- Arduino IDE

- Embedded C/C++

- Tinkercad Simulation

- I2C Communication

- PWM Digital Outputs

# Learning Outcomes

This project helped in understanding:

- Analog input handling

- Digital output control

- LCD interfacing using I2C

- Embedded systems logic

- Function-based code structuring

- Hardware simulation and debugging

- Circuit documentation

# Future Improvements

- RGB brightness control using PWM

- Rotary encoder navigation

- EEPROM memory saving

- Multiple LCD menu pages

- Bluetooth/WiFi control using ESP32

- RTOS-based task handling

- Sensor integration

# Project Status

- Simulation Completed

- Hardware Tested

- Fully Functional

- GitHub Uploaded

- Self-Designed Logic

# Author

Aditya Maindargi

Manufacturing Science & Engineering

COEP Technological University